KWARA STATE UNIVERSITY

DEPARTMENT OF AERONAUTICAL AND ASTRONAUTICAL  ENGINEERING

KWASU AAE : 302 AEROSPACE LABORATORY TECHNOLOGY

EXPERIMENT 5: LABORATORY REPORT ON DRAG FORCE MEASUREMENT ON CYLINDRICAL BODIES USING  AIRFLOW BENCH AF10 DRAG APPARATUS

OSOKOYA OLUWABUKUNMI OLADAPO

23/67AA/544

JUNE 4, 2026

ABSTRACT

This laboratory report details the experimental determination of the coefficient of drag (Cd) for a circular cylindrical body exposed to a steady, incompressible airflow within a subsonic test section. The primary objective was to investigate the structural and fluid-dynamic behavior of a cylinder by monitoring the relationship between measured drag force and variation in dynamic pressure. Experiments were executed by acquiring differential manometric data—specifically total pressure (Po) and static pressure (po)—at discrete velocity configurations, while simultaneously recording the generated drag forces via a calibrated balance system. The raw force values were corrected for a structural tare cylinder mass of 17.30 gmf, and manometric tares were applied to account for initial system offsets. Air density was computed as 1.1540 kg/m3 utilizing the ideal gas equation under local thermodynamic parameters. A linear regression framework established an exceptional correlation coefficient (R2 = 0.9955) between the dynamic pressure and the drag force in Newtons, confirming classical aerodynamic scaling laws. From the empirical slope of the linear fit and the structural projected frontal area (A = 6.00 x 10-4m2), the experimental coefficient of drag was isolated as 0.8594. This result is analyzed relative to standard theoretical models, boundary layer regimes, and specific flow separation patterns over smooth cylinders, with secondary consideration given to experimental uncertainties such as test section blockage effects and high-frequency instrumentation noise.

I

INTRODUCTION

The aerodynamic characterization of bluff bodies immersed in fluid streams represents a foundational domain within classical fluid dynamics and aerospace engineering. Unlike streamlined aerodynamic structures, such as specialized low-drag airfoils, bluff bodies exhibit a geometric profile where the characteristic structural length scale perpendicular to the oncoming flow is comparable to or greater than the length parallel to the flow. A circular cylinder is an archetypal bluff body that experiences significant pressure drag due to the massive flow separation and subsequent low-pressure wake generated downstream of the body. Understanding the mechanical distribution of forces across a cylindrical topology is highly relevant for mitigating structural loads on components such as landing gears, external struts, missiles, and marine risers.

When an unseparated fluid stream approaches a circular cylinder, it creates a high-pressure stagnation point at the forward vertex. As the fluid accelerates over the curved surface, local static pressure drops to a minimum while velocity peaks. In inviscid, ideal flow fields, D'Alembert’s Paradox states that symmetrical pressure distribution results in zero net drag. However, in real viscous fluids, the growth of a  boundary layer exposed to a severe adverse pressure gradient over the rear half of the cylinder induces fluid deceleration and eventual flow separation. This separation detaches the shear layers, yielding a highly turbulent, broad wake characterized by a pressure significantly lower than the stagnation pressure. The resulting asymmetric pressure profile dominates the total drag force, a phenomenon known as pressure or form drag.

System identification and empirical curve-fitting play an essential role in validating mathematical models with real-world physics. By quantifying the aggregate force through direct measurement and mapping it against the fluid’s kinetic energy per unit volume—defined as the dynamic pressure—aerodynamicists isolate non-dimensional scaling coefficients. These coefficients allow experimental results obtained from scale models to be accurately extrapolated to full-scale aerospace structures under identical non-dimensional flow characteristics.

OBJECTIVES

The primary engineering objectives of this experimental investigation are structured as follows:

- To acquire precise empirical aerodynamic drag force measurements on a circular cylindrical body under varied subsonic flow regimes using a laboratory Air flow bench apparatus.

- To capture and evaluate differential pressure data via total and static manometric pressure measurements to compute the localized dynamic pressure.

- To establish a reliable linear regression framework relating experimental drag force to dynamic pressure, validating the fundamental aerodynamic equation.

- To compute the empirical coefficient of drag (CD) for the circular cylinder using the isolated slope of the linear model and the projected frontal area of the body.

- To assess structural, environmental, and instrumentation factors that contribute to variance between experimental coefficients and classical infinite-cylinder benchmarks.

EXPERIMENTAL SETUP

The laboratory apparatus utilized for this experiment comprised an open-circuit, low-speed subsonic test section engineered for aerodynamic visualization and force characterization. The test section featured clean transparent walls allowing unhindered viewing of the mounted test article. The specimen was a rigid, smooth circular cylinder with a precisely machined diameter (d) of 0.0125 meters and an exposed span length (L) of 0.048 meters within the flow stream. The cylinder was securely anchored to an external vertical cantilever load-sensing frame coupled with an analogue force transducer and a digital display configured to output force in gram-force (gmf).

Pressure metrics within the test section test section were acquired utilizing a dual pitot-static manometric arrangement. A total pressure probe (Po) was aligned parallel to the oncoming free-stream vector to register total pressure, while a flush-mounted wall static tap recorded the static pressure reference (po). Both lines were plumbed directly to a multi-tube liquid inclined manometer calibrated in millimeters of water column (mmH20). Environmental monitoring equipment included a digital thermometer to record fluid temperature continuously across the operational cycles.

METHODOLOGY

The experimental procedures and operational sequences followed during this investigation were executed as documented in A First Course in Airflow by Emeritus Professor E. Markland. Initial instrument baselines were recorded prior to starting the test section fan to establish mathematical datums under zero-flow conditions to prevent future instrument misalignment . The baseline structural tare weight offset (cylinder_mass) due to the mounting assembly was recorded as 17.30 gmf. The initial manometric levels revealed zero-velocity tare offsets of  Po = 136 mmH20 for total pressure and po = 140 mmH20 for static pressure. Following baseline calibration, the test section propulsion motor was energized. The flow control valve was systematically varied across six distinct increments to sweep a robust range of dynamic pressures. At each discrete velocity step, the fluid was permitted to reach steady-state equilibrium before logging the ambient fluid temperature, total pressure height (Po), static pressure height (po), and the gross indicated drag force. The compiled data matrix was subsequently saved for reduction, algorithmic processing, and curve-fitting.

RESULT

The reduction of the raw experimental dataset necessitated systematic correction and unit conversion to ensure dimensional homogeneity within the International System of Units (SI). Environmental density computation was executed by analyzing the measured temperature data array T_celcius = [30, 33, 33, 33, 33, 33], which yielded an average fluid temperature T_avg of 32.50 °C to 305.65 K. Utilizing an absolute atmospheric reference pressure p of 1.013 105 N/m and the specific gas constant for dry air R = 287.2 J/kg•K , the ambient fluid density was governed by the ideal gas law:

ρ =pR ⋅T

Where:

ρ  =  Air density in kilograms per cubic meter (kg/m3)

p  =  Absolute static atmospheric pressure (1.013×105 N/m2)

R  =  Specific gas constant for dry air (287.2 J/kg⋅K)

T  =  Absolute thermodynamic temperature in Kelvin (K)

P = ρ • g • h

Conversion : mmH2O to N/m²

- The pressure equation P = ρ · g · h evaluates the force exerted by a column of fluid per unit area.

- For a unit height of h = 1 mmH2O, the value is converted to base SI units as h = 0.001 meters.

- Standard density of water at peak purity and 4°C is defined as ρ = 1,000 kg/m³.

- Standard acceleration due to gravity is defined by international agreement as g = 9.80665 m/s².

- Substituting these physical values yields the equation: P = (1,000 kg/m³) · (9.80665 m/s²) · (0.001 m).

- Simplifying the numerical values and units yields P = 9.80665 kg/(m·s²), representing standard pressure.

- Because 1 N = 1 kg·m/s², the unit kg/(m·s²) directly maps to N/m², proving that 1 mmH2O = 9.80665 N/m².

Substituting the empirical thermodynamic variables into Equation 1 yielded an operational air density (ρ)  of  1.1540 kg/m3. Raw manometric readings obtained in mmH2O were systematically corrected by subtracting their respective baseline no-flow tares (Po,corr = Po− 136;  po,corr = po − 140). The true differential manometric height head (ΔP) was subsequently determined. To transform this water-height differential into an equivalent SI unit of pressure (Pascal or N/m2), the conversion scale of  9.81 N/m2per mmH2O was applied, thus yielding the true dynamic pressure (q = ΔP × 9.81). Concurrently, the raw drag forces were isolated from structural tares (Dgmf  = Draw − 17.30) and converted to Newtons using the gravitational metric relation where 1 gmf equals 0.00980665 N. The complete, fully processed engineering dataset is structured and presented in Table 1.

| S/N | Po (mmH20) | po (mmH20) | △P = Po - po Pressure Head | △P (N) | Raw Force (gmf) | Net Force (gmf) | Net Drag Force (N) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 138 | 142 | 0 | 0.00 | 19.27 | 1.97 | 0.01932 |
| 2 | 144 | 142 | 6 | 58.86 | 21.30 | 4.00 | 0.03923 |
| 3 | 154 | 144 | 14 | 137.34 | 25.33 | 8.03 | 0.07875 |
| 4 | 168 | 146 | 26 | 255.06 | 30.69 | 13.39 | 0.13131 |
| 5 | 180 | 150 | 34 | 333.54 | 36.49 | 19.19 | 0.18819 |
| 6 | 206 | 154 | 56 | 549.36 | 47.63 | 30.3 | 0.29744 |

Table 1: Processed Experimental Parameters and Aerodynamic Data Matrix

Fundamental fluid mechanics states that the total drag force sustained by an immersed body is directly proportional to the dynamic pressure of the oncoming free-stream fluid, written as:

D = CD ⋅ A ⋅ q

Where:

D  =  Aerodynamic Drag Force in Newtons (N)

CD  =  Dimensionless Coefficient of  Drag

A  =  Projected frontal cross-sectional area (m2)

q  =  Dynamic pressure ( 12ρU 2) in N/m2)

Here, A is the projected frontal area defined by the multiplication of cylinder diameter and span length (A = d ⋅ L). For the given geometry, A  = 0.0125 m × 0.048 m =6.00×10−4 m2.

A linear first-degree polynomial curve-fit was executed to map the target variable D as a function of the independent variable q  according to the linear model equation y = mx + c.

The mathematical model derived from empirical regression is expressed:

D = 0.0005156 q + 0.00110

The statistical goodness-of-fit was verified via the coefficient of determination, yielding an exceptional linearity of R2=0.9955. (Note: The corresponding plot represents a near-perfect linear alignment crossing the Y-axis at a subtle mechanical intercept of  0.0110 N).

By matching the experimental regression equation coefficients to the analytical expression in Equation 2, the empirical slope (m) directly corresponds to the product of the drag coefficient and the projected area (m = CD ⋅ A). Consequently, the non-dimensional coefficient of drag is extracted via the relation:

CD  =slopeA=0.00051560.000600  = 0.8594

Figure CD: A linear regression fit graph of drag force against dynamic pressure

DISCUSSION

In this experiment, the drag coefficient (a number that measures an object's air resistance) of a circular cylinder was tested at moderate airspeeds. Textbook physics for a perfectly smooth, infinitely long cylinder says this number should hover between 1.0 and 1.2. However, the lab test resulted in a lower value of 0.8594, meaning the model actually experienced less wind resistance than expected. This difference comes down to the real-world shape of the cylinder and the limitations of the lab equipment. The main reason for the lower drag is a phenomenon called the "end-flow effect." Textbooks assume air flows perfectly over a cylinder that never ends. In the lab, however, the cylinder is short and stubby. Because the ends of the cylinder are exposed, the high-pressure air hitting the front can easily escape around the sides and slip into the low-pressure pocket (the wake) behind it. This extra airflow around the edges balances out the pressure differences, shrinks the turbulent wake behind the cylinder, and ultimately reduces the total drag. Another interesting finding was that the testing equipment recorded a small amount of drag force even when the test section was completely turned off. Theoretically, no wind should mean zero drag. This error happened because of minor mechanical and electrical issues. The pivot and sensor holding the cylinder had a tiny bit of physical friction (called "stiction"), which stopped it from settling perfectly back to zero after being pushed by the wind. On top of that, the electronic sensors experienced a slight voltage drift, causing a small mathematical offset in the data. Finally, everyday lab environment issues introduced a few extra errors. Vibrations from the test section's heavy electric motor shook the testing stand, making the digital screen fluctuate and forcing researchers to guess a visual average. At very slow speeds, the liquid pressure gauges were hard to read accurately due to visual angles and the natural curve of the liquid. Lastly, because the cylinder took up physical space inside the narrow test section, it squeezed the oncoming air and forced it to speed up around the sides, which artificially raised the pressure readings compared to what the tunnel walls recorded.

CONCLUSION

This experimental study successfully investigated the aerodynamic drag characteristics of a circular cylinder exposed to varying dynamic pressures in a subsonic test section. All core engineering objectives were systematically achieved. Using a linear regression model, a clear mathematical relationship was established between the corrected drag force (N) and the dynamic pressure (N/m2), demonstrating excellent data linearity as confirmed by a coefficient of determination (R2) of 0.9955. The non-dimensional experimental coefficient of drag (CD) was successfully isolated as 0.8594. The slight variance between this empirical value and classical infinite-cylinder benchmarks was thoroughly analyzed and attributed to three-dimensional end-flow relief caused by the low aspect ratio (AR = 3.84) of the scale model, instrument hysteresis, and tunnel wall blockage effects. Ultimately, this experiment confirmed the square-law velocity scaling of aerodynamic drag on bluff bodies and highlighted the practical complexities of experimental aerodynamics.

APPENDIX

Air Density

p = 1.013e5; % N/m2

R = 287.2 ; % Gas constant

T_celcius = [30,33,33,33,33,33]; % Measured Temperatures in celcius

temp_celcius = mean(T_celcius) % Average temperature

temp_celcius = 32.5000

T = 273.15 + temp_celcius; % Temperature in kelvin

rho = p/(R*T) % Air density in kg/m³

rho = 1.1540

Table of Results

cylinder_mass = 17.30;

initial_Po = 136;

initial_po = 140;

drag_force = [19.27,21.30,25.33,30.69,36.49,47.63]; % Drag Force Measurements in gram-force

Po = [138,144,154,168,180,206]; % total pressure

po = [142,142,144,146,150,154]; % static pressure

Po = Po - initial_Po;

po = po - initial_po;

drag_force = drag_force - cylinder_mass;

% The drag force is in 'gmf' (gram-force). It MUST be converted to Newtons (N)

% to keep the equation dimensionally homogeneous with dynamic pressure (N/m^2).

% 1 gmf = 0.00980665 N (or ~9.81e-3 N) also 1 N = 101.9716212978 gmf

drag_force_Newton = drag_force * 0.00980665;

delta_P = Po - po;

dynamic_pressure = delta_P .* 9.81; % conversion to SI unit N/m²

Drag_Force_Experiment = table(drag_force',drag_force_Newton',Po',po',delta_P',dynamic_pressure',T_celcius'); % Data Table

Drag_Force_Experiment.Properties.VariableNames = ["Drag Force (gmf) ","Drag Force (N)","Po (mmH20)","po (mmH20)"," Po - po = 1/2ρU²","Dynamic Pressure (N/m²)","T/°C"];

Drag_Force_Experiment

Drag_Force_Experiment = 6×7 table

The Graph of Drag Force VS Dynamic Pressure

scatter(dynamic_pressure,drag_force_Newton)

grid on

xlabel("Dynamic Pressure 1/2ρU² (N/m²)")

%ylabel("Drag Force D1 (gmf) ")

ylabel("Drag Force (N)")

title("Experiment to measure drag force on a circular cylinder")

hold on

Fitting the data

Linear Fit

% Shorten variables for convenience

x = dynamic_pressure;

y = drag_force_Newton;

% Generate a smooth array of x-values for high-resolution curve plotting

x_fine = linspace(min(x), max(x), 200);

% 2. Curve Fitting Calculations

p_lin = polyfit(x, y, 1); % 1st degree polynomial

y_fit_lin = polyval(p_lin, x); % Values at data points

y_curve_lin = polyval(p_lin, x_fine); % Values for smooth line

% 3. Goodness of Fit Assessment (R-squared)

ss_tot = sum((y - mean(y)).^2); % Total Sum of Squares

R2_lin  = 1 - (sum((y - y_fit_lin).^2) / ss_tot);

fprintf('Linear:      y = %.5fx + %.4f  (R^2 = %.4f)\n', p_lin(1), p_lin(2), R2_lin);

Linear: y = 0.00052x + 0.0110 (R^2 = 0.9955)

Plot of Fitted Curve

plot(x, y_fit_lin, 'r-', 'LineWidth', 2, 'DisplayName', sprintf('Linear Fit (R^2 = %.4f)', R2_lin));

Coefficient of drag determination

% polyfit returns coefficients in descending powers: p = [slope, intercept]

coefficients = polyfit(x, y, 1);

slope = coefficients(1);

fprintf('The calculated slope is: %.5f\n', slope);

The calculated slope is: 0.00052

%% 3. Calculate Coefficient of Drag (CD)

d = 0.0125; % Cylinder diameter in meters

L = 0.048; % Cylinder length/span in meters

A = d * L; % Frontal projected area (m^2)

% Calculate CD: Slope = CD * A  =>  CD = Slope / A

CD = slope / A;

fprintf('The calculated Coefficient of Drag (CD) is: %.4f\n', CD);

The calculated Coefficient of Drag (CD) is: 0.8594

REFERENCES

- John D. Anderson Jr.(2017). Fundamentals of Aerodynamics Sixth edition New York, NY: McGraw Hill Education 2017.

- Emeritus Professor E. Markland. (2009). A first course in airflow. Hooton Street Nottingham: TecEquipment Ltd 2009.

- MathWorks. (2026). polyfit: Polynomial curve fitting. MATLAB Documentation. https://www.mathworks.com/help/matlab/ref/polyfit.html